DIACOM ®  Diagnostic Status Report

08/08/2004 10:29:27

1. VEHICLE CONFIGURATION

Engine Make:   		502 EFI

Engine Serial No:		N/A

Model Year:     		1993

System Type:   		GM-DELCO MEFI 1 MODE 1 TWIN

2. PERFORMANCE DATA

Master Data

Parameter Description              	Value               	Unit

Baro Sensor Voltage                	4,76                	VDC

Base IAC Position                  	99                  	#

Battery Voltage                    	13,80               	VDC

Calibration Checksum               	9550                	HEX

Closed Loop Status                 	Off

Coolant Overtemp Protection        	Yes

Coolant Temperature                	56                  	°C

Desired IAC Position               	163                 	#

Desired Idle Speed                 	644                 	RPM

ECM Program ID                     	B7                  	HEX

Emergency Stop Enabled             	No

Emergency Stop Input               	Off

Engine Overtemp Detect             	Off

Engine Running Time                	5,53                	Min

Engine Speed                       	5041                	RPM

Fuel Flow Rate                     	98,77               	LPH

Fuel Mixture Status                	Lean

High Trans Temp Detect             	Off

High Trans Temp Protection         	Yes

I/O Fluid Level Light              	No

I/O Fluid Level Protection         	No

IAC Position                       	150                 	#

IAC Throttle Follower              	64                  	#

Inj Pulse Width Cranking           	11,10               	ms

Injector Pulse Width               	9,00                	ms

Knock Detonation Control           	Yes

Knock Retard                       	0,00                	DEG

Low I/O Fluid Detect               	Off

Low Oil Level Detect               	Off

Low Oil Level Protection           	No

Low Oil Pressure Detect            	Off

Low Oil Pressure Protection        	Yes

Low Water Flow Detect              	Off

Low Water Flow Protection          	No

Manifold Absolute Pressure         	98                  	KPA

Manifold Air Temperature           	40                  	°C

MAP Sensor Voltage                 	4,61                	VDC

Over Trim Detect                   	Off

Oxygen Sensor Voltage              	0                   	mV

Power Reduction Mode               	Off

Prime Fuel Pump On Keyup           	Yes

Quasi-Sync Fuel Mode               	Off

Single TBI System                  	No

Spark Advance                      	31                  	DEG

Throttle Follower Bias             	255                 	#

Throttle Position                  	100                 	%

Throttle Sensor Voltage            	4,39                	VDC

Total Trouble Codes                	0

Trim Angle                         	0                   	%

Trim Protection                    	No

Slave Data

Parameter Description              	Value               	Unit

Baro Sensor Voltage                	4,76                	VDC

Base IAC Position                  	47                  	#

Battery Voltage                    	13,70               	VDC

Calibration Checksum               	9550                	HEX

Closed Loop Status                 	Off

Coolant Overtemp Protection        	Yes

Coolant Temperature                	72                  	°C

Desired IAC Position               	111                 	#

Desired Idle Speed                 	600                 	RPM

ECM Program ID                     	B7                  	HEX

Emergency Stop Enabled             	No

Emergency Stop Input               	Off

Engine Overtemp Detect             	Off

Engine Running Time                	5,82                	Min

Engine Speed                       	4795                	RPM

Fuel Flow Rate                     	94,74               	LPH

Fuel Mixture Status                	Lean

High Trans Temp Detect             	Off

High Trans Temp Protection         	Yes

I/O Fluid Level Light              	No

I/O Fluid Level Protection         	No

IAC Position                       	111                 	#

IAC Throttle Follower              	64                  	#

Inj Pulse Width Cranking           	10,90               	ms

Injector Pulse Width               	9,00                	ms

Knock Detonation Control           	Yes

Knock Retard                       	0,00                	DEG

Low I/O Fluid Detect               	Off

Low Oil Level Detect               	Off

Low Oil Level Protection           	No

Low Oil Pressure Detect            	Off

Low Oil Pressure Protection        	Yes

Low Water Flow Detect              	Off

Low Water Flow Protection          	No

Manifold Absolute Pressure         	98                  	KPA

Manifold Air Temperature           	35                  	°C

MAP Sensor Voltage                 	4,67                	VDC

Over Trim Detect                   	Off

Oxygen Sensor Voltage              	0                   	mV

Power Reduction Mode               	Off

Prime Fuel Pump On Keyup           	Yes

Quasi-Sync Fuel Mode               	Off

Single TBI System                  	No

Spark Advance                      	30                  	DEG

Throttle Follower Bias             	255                 	#

Throttle Position                  	100                 	%

Throttle Sensor Voltage            	4,25                	VDC

Total Trouble Codes                	0

Trim Angle                         	0                   	%

Trim Protection                    	No

3. TROUBLE CODE STATUS

Current ECM Codes

Master TCode

Slave TCode